Annex D.1 Template

Emergency and contingency procedure cards

| Field | Template entry |
| --- | --- |
| Operator / organisation | [Operator Name] |
| Operation / campaign | [Operation or Campaign Name] |
| Location / operating area | [General Operating Area] |
| Version | [vX.Y] |
| Date | [YYYY-MM-DD] |
| Prepared by | [Name / Role] |
| Reviewed by | [Name / Role] |

# How to use this template

This is a blank template for SORA-guided SAIL II field-supported BVLOS operations. It is intended as a starting point only. It does not by itself demonstrate regulatory compliance, replace an operational authorisation, or remove the need for review by competent personnel and the relevant aviation authority.

Before use, replace all bracketed placeholders, adapt the content to the aircraft, crew, operating environment, authority requirements, and approved risk assessment, and remove any example text that is not applicable.

# How to use these procedure cards

Adapt each card to the aircraft, automation level, C2 system, operating area, and approved emergency response plan. Keep cards short enough to use under workload.

# Loss or degradation of C2 link

| Field | Procedure content |
| --- | --- |
| Trigger | Telemetry loss, command delay, low RSSI, lost video/control, abnormal ground station state. |
| Immediate actions | Call: C2 issue. Remote pilot follows pre-briefed lost-link procedure. Assistant pilot reads C2 procedure and starts timing. Air observer increases traffic scan. Safety lead decides whether to pause adjacent operations. |
| Record required? | [Yes / No / condition] |
| External notification required? | [Yes / No / condition] |
| Return-to-operation rule | [Define who may approve resumption and what evidence is required.] |

# Crewed aircraft observed nearby

| Field | Procedure content |
| --- | --- |
| Trigger | Crewed aircraft visual/audio detection, ATC/radio warning, ADS-B/EC alert, observer report. |
| Immediate actions | Call: Traffic. Remote pilot descends, holds, lands, or executes approved separation action. Air observer maintains visual contact where possible. Assistant pilot records time, location, and action. Resume only after traffic is clear and safety lead confirms. |
| Record required? | [Yes / No / condition] |
| External notification required? | [Yes / No / condition] |
| Return-to-operation rule | [Define who may approve resumption and what evidence is required.] |

# Third-party or wildlife proximity

| Field | Procedure content |
| --- | --- |
| Trigger | Person, vehicle, livestock, wildlife, or unauthorised party enters the defined operating threshold. |
| Immediate actions | Call: Ground stop / wildlife stop as appropriate. Remote pilot holds, re-routes, climbs/descends, or lands according to procedure. Ground observer describes location and movement. Resume only when threshold is clear. |
| Record required? | [Yes / No / condition] |
| External notification required? | [Yes / No / condition] |
| Return-to-operation rule | [Define who may approve resumption and what evidence is required.] |

# Low battery or endurance margin

| Field | Procedure content |
| --- | --- |
| Trigger | Battery threshold reached, unexpected consumption, solar/power shortfall, headwind, extended recovery path. |
| Immediate actions | Call: Battery margin. Remote pilot initiates return or landing according to threshold. Assistant pilot confirms recovery path and landing area. Record deviation if threshold or recovery sequence differed from plan. |
| Record required? | [Yes / No / condition] |
| External notification required? | [Yes / No / condition] |
| Return-to-operation rule | [Define who may approve resumption and what evidence is required.] |

# Hard landing, impact, or suspected damage

| Field | Procedure content |
| --- | --- |
| Trigger | Hard landing, strike, abnormal sound, structural concern, payload impact, propeller/rotor strike. |
| Immediate actions | Disarm and make area safe. Do not relaunch until trigger-based inspection is complete. Record event and inspection outcome. Escalate if people, property, wildlife, or crewed aircraft were affected. |
| Record required? | [Yes / No / condition] |
| External notification required? | [Yes / No / condition] |
| Return-to-operation rule | [Define who may approve resumption and what evidence is required.] |