Annex D Template

Emergency response plan

| Field | Template entry |
| --- | --- |
| Operator / organisation | [Operator Name] |
| Operation / campaign | [Operation or Campaign Name] |
| Location / operating area | [General Operating Area] |
| Version | [vX.Y] |
| Date | [YYYY-MM-DD] |
| Prepared by | [Name / Role] |
| Reviewed by | [Name / Role] |

# How to use this template

This is a blank template for SORA-guided SAIL II field-supported BVLOS operations. It is intended as a starting point only. It does not by itself demonstrate regulatory compliance, replace an operational authorisation, or remove the need for review by competent personnel and the relevant aviation authority.

Before use, replace all bracketed placeholders, adapt the content to the aircraft, crew, operating environment, authority requirements, and approved risk assessment, and remove any example text that is not applicable.

# D.1 Purpose

This annex defines the response structure for contingencies, emergencies, escalation, and return-to-operation decisions.

# D.2 Response principles

Protect people first, then crewed aircraft and other airspace users, then property, wildlife, data, and equipment.

Use predefined calls and simple role actions where possible.

Escalate early when third-party safety, crewed aircraft, injury, property damage, fire, fly-away, or loss of control is involved.

Pause operations after an occurrence until the safety lead confirms the next step.

# D.3 Emergency roles

| Role | Emergency responsibility |
| --- | --- |
| Remote pilot | Aviate, maintain or regain control, execute emergency or termination procedure if required. |
| Assistant pilot | Read emergency checklist, monitor timing, support communication, record key events. |
| Safety lead / coordinator | Coordinate escalation, pause operations, decide recovery and resumption path. |
| Ground observer | Monitor people, vehicles, wildlife, and ground hazards; keep others clear. |
| Air observer | Monitor crewed traffic and airspace conflict; communicate traffic information. |
| External-service liaison | Contact ATC, site authority, emergency services, ranger/radio room, or equivalent services where required. |

# D.4 Emergency contact register

| Service / contact | Role in emergency | Availability | Contact method | Notes |
| --- | --- | --- | --- | --- |
| Local emergency services | [Medical / fire / police] | [Hours] | [Phone / radio] |  |
| Site authority / land manager | [Access / local response] | [Hours] | [Phone / radio] |  |
| ATC or airspace contact | [Airspace coordination] | [Hours] | [Phone / radio] |  |
| Internal safety lead | [Operator response] | [Hours] | [Phone / radio] |  |

# D.5 Event classification

| Event class | Examples | Minimum action |
| --- | --- | --- |
| Contingency | Low battery, minor link degradation, mission file issue, temporary ground obstruction. | Follow procedure, record if safety margin was affected. |
| Occurrence | Deviation from procedure, reduced safety margin, unexpected aircraft behaviour, near conflict. | Report using Annex E.1 and pause if needed. |
| Emergency | Injury, collision, uncontrolled fly-away, fire, loss of control near third parties, crewed aircraft conflict. | Activate emergency response, escalate externally where required, report. |

# D.6 Tabletop exercise record

| Scenario | Participants | Gaps identified | Action owner | Due date |
| --- | --- | --- | --- | --- |
| Loss of C2 link |  |  |  |  |
| Crewed aircraft conflict |  |  |  |  |
| Third-party intrusion |  |  |  |  |
| Crash or hard landing |  |  |  |  |